# Project #3 – Add Motor

Document reference: Lamb, Shaun.cmpe311.spring26.Project#3-CE

Date of publication: March 28th, 2026

LEAD ENGINEER: Shaun Lamb, University of Maryland, Baltimore County

STAKEHOLDERS:
Prof Kidd, Instructor, University of Maryland, Baltimore County, Baltimore, Maryland, USA
MD Safwan Zaman, TA, University of Maryland, Baltimore County, Baltimore, Maryland, USA

## HIGH-LEVEL DESCRIPTION

This is the project document for Project 3 of the CMPE 311 class, Spring 2026. It contains all relevant testing requirements for project 3, which extends projects 1 and 2 by adding a DC motor-driven fan controlled by a PWM signal. This document outlines the updated customer requirements, high-level technical specifications, system design, and testing procedures and scenarios. It also presents the results of testing.

This design meets the same customer and high-level requirements as Project #1&2 while satisfying the additional Educational Requirement to implement a function-pointer-based cyclic executive.

## REFERENCES

Ref: ProjectAddMotor – The definition of the project this document addresses
Ref: kiddKid.cmpe311.fall25.project#1 – Asynchronous Execution
Ref: projectTaskMgmtCE_draft.docx – Educational Requirement Specification
Ref: Project 1 – Submission.docx – Customer and high-level requirement definitions
Ref: Project 2- Submission.docx
Ref: Elegoo UNO R3 datasheet – Hardware platform reference

## DEFINITIONS AND ABBREVIATIONS

Additional term for this document:
The User – Person operating The System.
The Developer – Person(s) designing and building The System.
The Customer – Person(s) paying for The System.
The Evaluator – Person(s) determining whether The System satisfies The Customer's requirements.
The Company – Organization contracted to build The System.

System and Document Terms:
The System – The embedded system being designed.
The Reference-platform – The ELEGOO Uno R3 development board.
The Contract – Business document legally binding The Company to deliver The System.
The Customer-requirements – Requirements defined by The Customer as satisfying The Contract.
The Requirements – High-level technical requirements derived from The Customer-requirements.
The Educational-constraints – Requirements imposed by the instructor, unrelated to The System.
ECR/ECN – Engineering Change Request/Note; document describing design changes. serial-monitor – Arduino IDE serial port used to communicate with The User.
See Project#1 – Asynchronous Execution for the complete glossary.

CUSTOMER REQUIREMENTS

## C.1 The User must be able to set the blink rate of two different LEDs.
C.2 The User must be able to update the blink rate of each of the LEDs independently.
C.3 Each LED must continue blinking at the last-set rate until updated by the User.
C.4 The System must run upon an Arduino Uno R3 compatible development board.
C.5 The blink rate of an LED must be expressed in terms of milliseconds.
C.6 The device must have a motor-driven fan. (added – Lamb, Shaun, 4/16/2026)
C.7 The RPM of the fan must be controllable via a button. (added – Lamb, Shaun, 4/16/2026)
C.8 The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH. (added – Lamb, Shaun, 4/16/2026)
C.9 Each hit of the button must cycle the speeds from OFF to LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF. (added – Lamb, Shaun, 4/16/2026)
C.10 LOW must be able to move air with minimal sound and minimum feeling of flow. (added – Lamb, Shaun, 4/16/2026)
C.11 MEDIUM must be able to move air with moderate sound and a moderate feeling of air flow. (added – Lamb, Shaun, 4/16/2026)
C.12 HIGH must be able to move air quickly with moderate sound and a distinct feeling of air flow. (added – Lamb, Shaun, 4/16/2026)

HIGH-LEVEL TECHNICAL REQUIREMENTS

HL.1 The System must use two LEDs connected to pins 2 and 3 of the Elegoo UNO R3.
HL.2 The System must execute under a Cyclic Executive dispatcher implemented as a function-pointer array.
HL.3 The System must communicate with the User via the Arduino IDE serial monitor (9600 baud).
HL.4 The System must remain fully responsive; no blocking code or delays.
HL.5 The User must specify LED blink intervals in milliseconds.
HL.6 The blink rate of one LED must not affect or interfere with the other LED.
HL.7. The device must have a DC motor-driven fan with an operating voltage between 3V and 12V. (added – Lamb, Shaun, 4/14/2026)
HL.8. The fan must operate at a minimum between 1500 and 6500 RPM. (added – Lamb, Shaun, 4/14/2026)
HL.9. The fan must draw less than 4 Amps. (added – Lamb, Shaun, 4/14/2026)
HL.10. The fan must be driven by a PWM signal with a maximum voltage of 5VDC. (added -Lamb, Shaun, 4/14/2026)
HL.11. The fan must have a maximum dimension less than 4"x4"x3" (not including cage). (added – Lamb, Shaun, 4/14/2026)
HL.12. The RPM of the motor must be controllable using a single-pole, momentary-on,
normally-off push button. (added – Lamb, Shaun, 4/14/2026)
HL.13. The operation of the button and motor must have no noticeable effect upon the operation
of previously implemented I/O. (added – Lamb, Shaun, 4/14/2026)
HL.14. The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH. (added – Lamb, Shaun, 4/14/2026)
HL.15. Each hit of the button must cycle the speeds from OFF to LOW to MEDIUM to HIGH to
MEDIUM to LOW to OFF. (added – Lamb, Shaun, 4/14/2026)
HL.16. LOW must be able to move air with a sound volume less than 35dB and airflow levelsbetween 30-45 CFM. (added – Lamb, Shaun, 4/14/2026)
HL.17. MEDIUM must be able to move air with a sound volume less than 40dB and airflow levels between 45-70 CFM. (added – Lamb, Shaun, 4/14/2026)
HL.18. HIGH must be able to move air with a sound volume less than 50dB and airflow levels between 70-100 CFM. (added – Lamb, Shaun, 4/14/2026)

## EDUCATIONAL REQUIREMENTS

E.1 The embedded system must implement a Cyclic Executive Task Manager using a function-pointer array to dispatch all asynchronous tasks.
E.2 The test and validation requirements must include those from Project #1 plus any additional tests necessary to confirm proper CE operation.
E.3 The final report must be a formal design document following the Project #1 structure and conventions.
E.4 The Evaluator must be able to successfully have The System perform the dialog in Table 4 and Table 5. (added – Lamb, Shaun, 3/20/2030)

## ENGINEERING CHANGE REQUEST

ECR 0.1 – 1/1/2030

Requestor: Prof. Kidd

Description of change: The customer has requested to take the design of the previous project and add a DC motor-driven fan operated by a PWM signal, and made adjustments to the customer's high-level technical and educational requirements.

Reason for change: The customer has added new requirements (c6-12) requesting that the motor-driven fan have an adjustable speed through the use of a button. These requirements need additional hardware components and adjustments to the software.

## ENGINEERING CHANGE NOTE

Author: Shaun Lamb

Reference ECR: ECR 0.1 – 1/1/2030, Requestor: Prof. Kidd

Documents changed:

Lamb, Shaun.cmpe311.spring26.Project#3-Motor (this document) – New revision of the Project #2 design document. This document supersedes Project #2 – Submission.docx for the purposes of Project #3-MotorAdd.

Summary of changes:

1. DESIGN – 
Hardware- added N-channel MOSFET, 1N4007 flyback diode, 100μF smoothing capacitor, 9v battery, and a working motor 
Software -  added Fan speed for loop of off to low(26) to medium(38) to high(64) to medium(38) to low(26) and repeat.

2. CUSTOMER REQUIREMENTS – No change. Requirements C.1 through C.4 are identical to those in Ref: Lamb, Shaun. cmpe311.spring26.Project#1 – Asynchronous Execution, CUSTOMER REQUIREMENTS section. Added C8-C12(4/28/2026)

3. HIGH-LEVEL TECHNICAL REQUIREMENTS – HL.1, HL.2, HL.3, HL.4, HL.5, and HL.6 are unchanged. Added HL.7&8 (4/28/2026

## DESIGN

### DESIGN PRE-REQUISITES

• ELEGOO Arduino Uno R3 clone
• Arduino IDE 2.3.3 or better
• Breadboard
• Two LEDs
• DC motor / fan
• N-channel MOSFET (e.g. IRF540N)
• 1N4007 flyback diode
• 220Ω resistor (gate), 10kΩ resistor (pull-down)
• 100μF capacitor (smoothing)
• 9V battery + connector
• Momentary pushbutton
• Connective wires

## CODE

Check the code file in GitHub

## TESTING AND VALIDATION

All tests below were conducted on the UNO R3 hardware using two LEDs and 330 Ω resistors and a Motor 
 Testing conditions:
T.1 – Dialogue from serial monitor sample matches Table 4.
T.2 – Blink rate of each LED is independent and doesn't change until prompted.
T.3 – Blink rate is specified in milliseconds.
T.4 – Blink rate must be correct.
T.5 – The blink rate setting must be able to repeat at least 5 times.
T.6 – Button press sequence matches Table 5: OFF > LOW > MEDIUM > HIGH > MEDIUM >LOW > OFF. (added – Lamb, Shaun, 4/14/2026)
T.7 – Fan speed changes are reflected correctly in the serial monitor after each button press.(added – Lamb, Shaun, 4/14/2026)
T.8 – Fan operation does not interfere with LED blinking or serial monitor dialogue. (added –Lamb, Shaun, 4/14/2026)
T.9 – Fan LOW speed produces minimal airflow and sound. (added – Lamb, Shaun, 4/14/2026)
T.10 – Fan MEDIUM speed produces moderate airflow and sound. (added – Lamb, Shaun, 4/14/2026)
T.11 – Fan HIGH speed produces distinct airflow. (added – Lamb, Shaun, 4/14/2026)
T.12 – Button sequence must be able to repeat at least 3 full cycles. (added – Lamb, Shaun, 4/14/2026)

Tests Preformed
T1	Satisfied in the video
T2	Satisfied in the video
T3	Satisfied in the video
T4	Satisfied in the video
T5	Satisfied in the video
T6	Satisfied in the video
T7	Satisfied in the video
T8	Satisfied in the video
T9	Satisfied in the video
T10	Satisfied in the video
T11	Satisfied in the video
T12	Satisfied in the video
Through my testing, I can do all of it, but my motor is not working 
-I remade a video to demonstrate the couple of cases I missed, but I didn’t show again in the video hooking up the motor straight up to the battery, since we already talked about that.

## VIDEO

https://youtube.com/shorts/g1ex3chPTEI?si=nQ932JEC6rfjFD2i