# Project #2 – Task Management (Cyclic Executive)

Document reference: Lamb, Shaun.cmpe311.spring26.Project#2-CE

Date of publication: March 17th, 2026

LEAD ENGINEER: Shaun Lamb, University of Maryland Baltimore County

STAKEHOLDERS:
Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA
MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

## HIGH-LEVEL DESCRIPTION

This project implements a Cyclic Executive Task Manager for an embedded system on the Elegoo UNO R3 that performs identically to Project #1 – Asynchronous Execution. The system allows a user to independently set and update blink intervals for two LEDs via the Arduino IDE Serial Monitor. Each LED blinks continuously and independently at its last-set rate.

Unlike Project #1, which utilized asynchronous control logic via millis(), this project restructures the same behavior under a function-pointer-based Cyclic Executive (CE) dispatcher. The CE loops over an array of task functions—one per LED and one for serial input—executing them in a non-preemptive, round-robin fashion.

This design meets the same customer and high-level requirements as Project #1 while satisfying the additional Educational Requirement to implement a function-pointer-based cyclic executive.

## REFERENCES

Ref: kiddKid.cmpe311.fall25.project#1 – Asynchronous Execution
Ref: projectTaskMgmtCE_draft.docx – Educational Requirement Specification
Ref: Project 1 – Submission.docx – Customer and high-level requirement definitions
Ref: Elegoo UNO R3 datasheet – Hardware platform reference

## DEFINITIONS AND ABBREVIATIONS

See Project#1 – Asynchronous Execution for the complete glossary.

Additional term for this document:

Cyclic Executive (CE) – A fixed scheduling pattern that repeatedly dispatches tasks in a round-robin order via function pointers.
Task Function Array – An array of function pointers used by the CE to call each system task sequentially.

## CUSTOMER REQUIREMENTS

C.1 The User must be able to set the blink rate of two different LEDs.
C.2 The User must be able to update the blink rate of each of the LEDs independently.
C.3 Each LED must continue blinking at the last-set rate until updated by the User.
C.4 The System must run upon an Arduino Uno R3 compatible development board.

## HIGH-LEVEL TECHNICAL REQUIREMENTS

HL.1 The System must use two LEDs connected to pins 2 and 3 of the Elegoo UNO R3.
HL.2 The System must execute under a Cyclic Executive dispatcher implemented as a function-pointer array.
HL.3 The System must communicate with the User via the Arduino IDE serial monitor (9600 baud).
HL.4 The System must remain fully responsive; no blocking code or delays.
HL.5 The User must specify LED blink intervals in milliseconds.
HL.6 The blink rate of one LED must not affect or interfere with the other LED.

## EDUCATIONAL REQUIREMENTS

E.1 The embedded system must implement a Cyclic Executive Task Manager using a function-pointer array to dispatch all asynchronous tasks.
E.2 The test and validation requirements must include those from Project #1 plus any additional tests necessary to confirm proper CE operation.
E.3 The final report must be a formal design document following the Project #1 structure and conventions.

## ENGINEERING CHANGE REQUEST

ECR 0.1 – 1/1/2030

Requestor: Prof. Kidd

Description of change: Modify the design resulting from the high-level requirements defined in Lamb, Shaun.cmpe311.spring26.Project#1 – Asynchronous Execution to use a round-robin cyclic executive task manager. There is no change to the Customer Requirements or the High-Level Technical Requirements defined in Lamb, Shaun.cmpe311.spring26.Project#1 – Asynchronous Execution.

Reason for change: Results in a more reliable design. The new design will reduce labor and design costs for implementing future extensions.

## ENGINEERING CHANGE NOTE

ECN 0.1 – Implementing ECR 0.1

Author: Shaun Lamb

Reference ECR: ECR 0.1 – 1/1/2030, Requestor: Prof. Kidd

Documents changed:

Lamb, Shaun.cmpe311.spring26.Project#2-CE (this document) – New revision of the Project #1 design document. This document supersedes Project #1 – Submission.docx for the purposes of Project #2.

Summary of changes:

1. DESIGN – Implementation: The asynchronous millis()-based control flow from Project #1. This design organizes the same three tasks — taskSerialInput, taskBlinkLED1, and taskBlinkLED2 — into a function-pointer array (taskList[]) that loop() cycles through on every iteration. The result is a clean round-robin Cyclic Executive. All timing is still millis()-based within each task, and no delay() calls appear anywhere in the code."

2. CUSTOMER REQUIREMENTS – No change. Requirements C.1 through C.4 are identical to those in Ref: Lamb, Shaun. cmpe311.spring26.Project#1 – Asynchronous Execution, CUSTOMER REQUIREMENTS section.

3. HIGH-LEVEL TECHNICAL REQUIREMENTS – HL.1, HL.3, HL.4, HL.5, and HL.6 are unchanged. HL.2 has been updated to explicitly specify the CE dispatcher mechanism, consistent with ECR 0.1.

## DESIGN

### DESIGN PRE-REQUISITES

• Elegoo UNO R3 (Arduino-compatible ATmega328P)
• Arduino IDE v2.3.3 or later
• Two LEDs (any color)
• Two 330 Ω resistors (series limiters)
• Breadboard and jumper wires
• USB cable (for power and serial communication)

### CIRCUIT WIRING

| Signal | UNO Pin | Connection |
| --- | --- | --- |
| LED 1 (anode +) | D2 → 330 Ω → LED → GND | Red LED recommended |
| LED 2 (anode +) | D3 → 330 Ω → LED → GND | Green LED recommended |
| Common Ground | GND rail | Both LED cathodes share ground |

## CODE

/*

Project #2 – Task Management (Cyclic Executive)

Platform: Elegoo/Arduino UNO R3 (ATmega328P)

Serial Monitor: 9600 baud

Summary:

- Implements a non-preemptive, round-robin cyclic executive (CE)

using a function-pointer task array.

- Tasks:

1) taskSerialInput  : non-blocking serial line reader + UI state machine

2) taskBlinkLED1    : toggles LED1 according to its period

3) taskBlinkLED2    : toggles LED2 according to its period

- No delay(); all timing uses millis().

- User-entered interval (ms) is treated as the FULL period; each

ON and OFF lasts interval/2 milliseconds (matches P1 dialog).

Usage (Serial):

> What LED? (1 or 2)

> 2

> What interval (in msec)?

> 600

=> LED2 blinks ~300 ms ON, ~300 ms OFF

*/

#include <Arduino.h>

// ----------------------------- Pins -----------------------------

static const uint8_t LED_PIN_1 = 2;   // LED1 on D2

static const uint8_t LED_PIN_2 = 3;   // LED2 on D3

// ----------------------------- Timing Model ---------------------

// The "period" is the full blink period in milliseconds.

// Each LED toggles every halfPeriod = max(1, period/2).

// This matches the Project-1 test dialog: entering 600ms yields ~300ms ON + ~300ms OFF.

static volatile unsigned long periodMs_LED1 = 1000;  // default 1s period

static volatile unsigned long periodMs_LED2 = 1000;  // default 1s period

// Internal toggling state

static unsigned long lastToggle_LED1 = 0;

static unsigned long lastToggle_LED2 = 0;

static bool state_LED1 = LOW;

static bool state_LED2 = LOW;

// ----------------------------- Serial UI State ------------------

// Non-blocking line input buffer

static const size_t LINE_MAX = 32;

static char lineBuf[LINE_MAX];

static size_t lineLen = 0;

enum InputState : uint8_t {

ASK_LED = 0,

ASK_INTERVAL = 1

};

static InputState uiState = ASK_LED;

static int selectedLED = -1; // 1 or 2 after a valid LED entry

// ----------------------------- Helpers --------------------------

static inline unsigned long halfPeriodClamp(unsigned long period) {

// Ensure at least 1 ms half-period to avoid zero-interval edge cases.

unsigned long hp = period / 2;

return (hp == 0 ? 1UL : hp);

}

static void promptAskLED() {

Serial.println(F("What LED? (1 or 2)"));

}

static void promptAskInterval() {

Serial.println(F("What interval (in msec)?"));

}

static bool isAllDigits(const char* s) {

if (!s || !*s) return false;

for (const char* p = s; *p; ++p) {

if (*p < '0' || *p > '9') return false;

}

return true;

}

// Process a completed input line (no CR/LF included).

static void processLine(const char* line) {

switch (uiState) {

case ASK_LED: {

if ((strcmp(line, "1") == 0) || (strcmp(line, "2") == 0)) {

selectedLED = atoi(line);

uiState = ASK_INTERVAL;

promptAskInterval();

} else {

Serial.println(F("Invalid LED. Enter 1 or 2."));

promptAskLED();

}

} break;

case ASK_INTERVAL: {

if (!isAllDigits(line)) {

Serial.println(F("Invalid interval. Must be a positive integer in milliseconds."));

promptAskInterval();

return;

}

// Parse and clamp (0 is invalid)

unsigned long val = strtoul(line, nullptr, 10);

if (val == 0) {

Serial.println(F("Invalid interval. Must be > 0."));

promptAskInterval();

return;

}

if (selectedLED == 1) {

periodMs_LED1 = val;

Serial.print(F("LED1 period set to "));

Serial.print(periodMs_LED1);

Serial.println(F(" ms (ON/OFF ~half each)."));

} else if (selectedLED == 2) {

periodMs_LED2 = val;

Serial.print(F("LED2 period set to "));

Serial.print(periodMs_LED2);

Serial.println(F(" ms (ON/OFF ~half each)."));

} else {

// Should never happen; reset UI gracefully

Serial.println(F("Internal state error; resetting prompt."));

}

// Reset for next cycle

uiState = ASK_LED;

selectedLED = -1;

promptAskLED();

} break;

}

}

// ----------------------------- Tasks ----------------------------

// Task 1: Non-blocking serial input reader + UI state machine.

// - Accumulates characters only when available.

// - Handles Backspace/Delete, ignores CR, ends lines on LF.

// - Never blocks; safe inside cyclic executive.

void taskSerialInput(void) {

while (Serial.available() > 0) {

char c = (char)Serial.read();

if (c == '\r') {

// Ignore CR; wait for LF to terminate line

continue;

} else if (c == '\n') {

// End of line -> process

lineBuf[lineLen] = '\0';

if (lineLen > 0) {

processLine(lineBuf);

}

lineLen = 0; // reset buffer

} else if (c == 8 || c == 127) { // Backspace or Delete

if (lineLen > 0) {

lineLen--;

}

} else {

// Accept visible characters up to buffer limit

if (lineLen < (LINE_MAX - 1)) {

lineBuf[lineLen++] = c;

}

// else: silently drop extra chars to remain non-blocking

}

}

}

// Task 2: Blink LED1 according to its configured period.

void taskBlinkLED1(void) {

unsigned long now = millis();

unsigned long hp = halfPeriodClamp(periodMs_LED1);

if ((now - lastToggle_LED1) >= hp) {

state_LED1 = !state_LED1;

digitalWrite(LED_PIN_1, state_LED1 ? HIGH : LOW);

lastToggle_LED1 = now;

}

}

// Task 3: Blink LED2 according to its configured period.

void taskBlinkLED2(void) {

unsigned long now = millis();

unsigned long hp = halfPeriodClamp(periodMs_LED2);

if ((now - lastToggle_LED2) >= hp) {

state_LED2 = !state_LED2;

digitalWrite(LED_PIN_2, state_LED2 ? HIGH : LOW);

lastToggle_LED2 = now;

}

}

// ----------------------- Cyclic Executive -----------------------

typedef void (*TaskFunc)(void);

// Round-robin task list (order can be adjusted)

TaskFunc taskList[] = {

taskSerialInput,

taskBlinkLED1,

taskBlinkLED2

};

static const int NUM_TASKS = (int)(sizeof(taskList) / sizeof(taskList[0]));

void setup() {

pinMode(LED_PIN_1, OUTPUT);

pinMode(LED_PIN_2, OUTPUT);

digitalWrite(LED_PIN_1, LOW);

digitalWrite(LED_PIN_2, LOW);

Serial.begin(9600);

while (!Serial) { /* wait for native USB; UNO returns immediately */ }

Serial.println(F("System ready (Cyclic Executive)."));

promptAskLED();

}

void loop() {

// Non-preemptive round-robin dispatcher

for (int i = 0; i < NUM_TASKS; ++i) {

taskList[i]();

}

// No delay(); fast polling keeps everything responsive.

}



## TESTING AND VALIDATION

All tests below were conducted on the UNO R3 hardware using two LEDs and 330 Ω resistors. Tests  are carried over from Ref: Lamb, Shaun.cmpe311.spring26.Project#1 – Asynchronous Execution, TESTING AND VALIDATION section.
I tested setting LED1and LED2 Bink rate, Independent LED rate, Invalid LED selection, Zero Interval Entry, and Invalid LED interval

## APPENDIX A – LOGIC FLOW

Setup → Initialize LEDs, Serial → Start Cyclic Executive Loop →
taskSerialInput → taskBlinkLED1 → taskBlinkLED2 → Repeat

## APPENDIX B – DESIGN CODE

Refer to the provided Arduino sketch for Project #2 – Task Management (Cyclic Executive).

## VIDEO

https://youtu.be/TY_F4o9ZuGg?si=t0_Ml53wAaxkL6aY