Project #5

EEPROM Data Frames via Semaphore (PROJECT-USERTOS)

Document reference: Lamb, Shaun.cmpe311.spring26.Project#5-USERTOS

Date of publication: May 5th, 2026

LEAD ENGINEER: Shaun Lamb, University of Maryland Baltimore County

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA

MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

# HIGH-LEVEL DESCRIPTION

This document describes Project Five for CMPE 311 (Spring 2026). The project builds upon Project Four (PROJECT-USERTOS) by adding EEPROM-based data frame management to the existing FreeRTOS task architecture. Three 256-byte data frames are allocated from the ATmega328P's built-in EEPROM and a FreeRTOS counting semaphore with an initial count of three manages their allocation and release. All prior hardware configuration, customer requirements, and task behavior remain unchanged. Rather than adding a fifth task (which would overflow the ATmega328P's 2KB RAM), the EEPROM demo is implemented as a helper function called from Task_Serial when the user types 'e', demonstrating frame acquisition, write, read-back, and release on demand.

# REFERENCES

Ref: projectEepromSema.pdf – The definition of Project 5

Ref: Lamb, Shaun.cmpe311.spring26.Project#4-FreeRTOS – The previous project (direct predecessor)

Ref: 311projectFreeRTOS.pdf – Project 4 specification

Ref: projectTaskMgmtCE_draft.docx – Educational Requirement Specification

Ref: Project 1 – Submission.docx – Customer and high-level requirement definitions

Ref: Elegoo UNO R3 datasheet – Hardware platform reference

Ref: ATmega328P datasheet – EEPROM electrical characteristics

# DEFINITIONS AND ABBREVIATIONS

"RTOS" – Real-Time Operating System; an OS designed to process data in real time with deterministic scheduling.

"FreeRTOS" – A widely-used open-source RTOS kernel ported to AVR via the Arduino_FreeRTOS library.

"Task" – A FreeRTOS thread of execution.

"Mutex" – Mutual exclusion semaphore used to protect shared resources between tasks.

"Counting Semaphore" – A semaphore whose internal count tracks the number of available instances of a resource. A Take decrements the count (blocking when zero); a Give increments it.

"Data Frame" – An amount of contiguous data (256 bytes) that can be allocated for use by a task.

"EEPROM" – Electrically Erasable Programmable Read-Only Memory; non-volatile storage on the ATmega328P that retains data across power interrupts.

"vTaskDelay" – FreeRTOS API call to block a task for a given number of ticks, yielding CPU to other tasks.

"Tick" – The FreeRTOS time unit; on Arduino Uno with default settings approximately 15 ms per tick.

See Project#4 – for additional terms.

# CUSTOMER REQUIREMENTS

C.1 The User must be able to set the blink rate of two different LEDs.

C.2 The User must be able to update the blink rate of each of the LEDs independently.

C.3 Each LED must continue blinking at the last-set rate until updated by the User.

C.4 The System must run upon an Arduino Uno R3 compatible development board.

C.6 The device must have a motor-driven fan. (added – Lamb, Shaun, 4/16/2026)

C.7 The RPM of the fan must be controllable via a button. (added – Lamb, Shaun, 4/16/2026)

C.8 The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH. (added – Lamb, Shaun, 4/16/2026)

C.9 Each hit of the button must cycle the speeds from OFF to LOW, MEDIUM, HIGH, MEDIUM, LOW, OFF. (added – Lamb, Shaun, 4/16/2026)

C.10 LOW must be able to move air with minimal sound and minimum feeling of flow. (added – Lamb, Shaun, 4/16/2026)

C.11 MEDIUM must be able to move air with moderate sound and a moderate feeling of air flow. (added – Lamb, Shaun, 4/16/2026)

C.12 HIGH must be able to move air quickly with moderate sound and a distinct feeling of air flow. (added – Lamb, Shaun, 4/16/2026)

# HIGH-LEVEL TECHNICAL REQUIREMENTS

HL.1 The System must use two LEDs connected to pins 2 and 3 of the Elegoo UNO R3.

HL.2 The System must execute under a Cyclic Executive dispatcher implemented as a function-pointer array.

HL.3 The System must communicate with the User via the Arduino IDE serial monitor (9600 baud).

HL.4 The System must remain fully responsive; no blocking code or delays.

HL.5 The User must specify LED blink intervals in milliseconds.

HL.6 The blink rate of one LED must not affect or interfere with the other LED.

HL.7 The device must have a DC motor-driven fan with an operating voltage between 3V and 12V. (added – Lamb, Shaun, 4/16/2026)

HL.8 The fan must operate at a minimum between 1500 and 6500 RPM. (added – Lamb, Shaun, 4/16/2026)

HL.9 The fan must draw less than 4 Amps. (added – Lamb, Shaun, 4/16/2026)

HL.10 The fan must be driven by a PWM signal with a maximum voltage of 5 VDC. (added – Lamb, Shaun, 4/16/2026)

HL.11 The fan must have a maximum dimension less than 4x4x3 inches (not including cage). (added – Lamb, Shaun, 4/16/2026)

HL.12 The RPM of the motor must be controllable using a single-pole, momentary-on, normally-off push button. (added – Lamb, Shaun, 4/16/2026)

HL.13 The operation of the button and motor must have no noticeable effect upon the operation of previously implemented I/O. (added – Lamb, Shaun, 4/16/2026)

HL.14 The fan must have 4 speeds: OFF, LOW, MEDIUM and HIGH. (added – Lamb, Shaun, 4/16/2026)

HL.15 Each hit of the button must cycle the speeds from OFF to LOW to MEDIUM to HIGH to MEDIUM to LOW to OFF. (added – Lamb, Shaun, 4/16/2026)

HL.16 LOW must be able to move air with a sound volume less than 35 dB and airflow levels between 30–45 CFM. (added – Lamb, Shaun, 4/16/2026)

HL.17 MEDIUM must be able to move air with a sound volume less than 40 dB and airflow levels between 45–70 CFM. (added – Lamb, Shaun, 4/16/2026)

HL.18 HIGH must be able to move air with a sound volume less than 50 dB and airflow levels between 70–100 CFM. (added – Lamb, Shaun, 4/16/2026)

HL.19 The system and application software will be built upon a RTOS (Real-time Operating System). (added – Lamb, Shaun, 4/1/2026)

HL.21 A mechanism must exist whereby tasks can temporarily save sensitive data, even across power interruptions. (added – Kidd, 4/20/2030)

# MID-LEVEL TECHNICAL REQUIREMENTS (for HL.21)

ML.21.2 A data frame must be at least 256 bytes.

ML.21.3 All data frames must be of the same size and indistinguishable from each other.

ML.21.4 There must be at least 3 such data frames, allocated from the EEPROM.

ML.21.5 Such data frames must be considered resources and managed as such by the task manager.

ML.21.6 The Task Manager must manage their allocation using a semaphore design pattern.

# EDUCATIONAL REQUIREMENTS

E.1 The Developer must use the version of gcc used by Arduino IDE 2.3.3 or later for the AVR ATmega328p.

E.2 The Developer must have any software used in The System hosted on a GitHub repository.

E.2.1 The Evaluator must be able to (a) download from the repository any software, documentation, and other material; (b) cross-compile; and (c) run on The Reference Platform.

E.3 LEDs must be connected to Arduino digital Pins 2 & 3.

E.4 The Evaluator must be able to successfully have The System perform the dialog in Table 5 and Table 6. (added – Kidd, 3/20/2030)

E.4.5 The Evaluator must be able to demonstrate the successful allocation and release of data frames by multiple tasks. (added – Kidd, 4/20/2030)

E.5 Verify that all previous tests still work for the current project. (added – Lamb, 4/1/2026)

# ENGINEERING CHANGE NOTE

ECN 5.1 – Implementing ECR 5 (4/20/2030)

Author: Shaun Lamb

Reference ECR: ECR 5 – 4/20/2030, Requestor: Kidd, Head Technical Lead

Documents changed:

Lamb, Shaun.cmpe311.spring26.Project#5-USERTOS (this document) – New revision of the Project #4 design document. This document supersedes Project#4 for the purposes of Project #5.

Summary of changes:

1. DESIGN – Added EEPROM data frame infrastructure (initFrames, acquireFrame, releaseFrame, writeFrame, readFrame) and a runEepromDemo helper function callable from Task_Serial via the 'e' command. A FreeRTOS counting semaphore (xFrameSema, initial count = 3) manages access to three 256-byte EEPROM frames. A fifth task was not added due to the ATmega328P's 2KB RAM constraint; the demo runs within Task_Serial's existing stack instead. The fan debounce logic was also corrected so that lastBtn is frozen during the debounce window, preventing ghost presses.

2. CUSTOMER REQUIREMENTS – No change.

3. HIGH-LEVEL TECHNICAL REQUIREMENTS – Added HL.21.

4. MID-LEVEL TECHNICAL REQUIREMENTS – Added ML.21.2 through ML.21.6 refining HL.21.

5. EDUCATIONAL REQUIREMENTS – Added E.4.5.

# DESIGN

## DESIGN PRE-REQUISITES

All hardware prerequisites from Project #4 remain unchanged. No additional hardware is required for Project #5, as the ATmega328P’s built-in 1 KB EEPROM is used.

- Elegoo UNO R3 (Arduino-compatible ATmega328P)

- Arduino IDE v2.3.3 or later with Arduino_FreeRTOS library

- Two LEDs with 330 Ω current-limiting resistors

- DC motor / fan, N-channel MOSFET (IRF540N), 1N4007 flyback diode, 220 Ω gate resistor, 10 kΩ pull-down resistor, 100 μF smoothing capacitor, 9 V battery

- Momentary push button, breadboard, jumper wires, USB cable

## DESIGN OVERVIEW

The Project #4 four-task FreeRTOS architecture is retained without modification. Task_Serial (Priority 2) now also handles the 'e' EEPROM demo command in addition to its existing LED interval dialog. Task_LED1, Task_LED2, and Task_Fan remain at Priority 1 and are unchanged in behavior. A second semaphore, xFrameSema (counting, initial count = 3), is created alongside xSerialMutex. The fan debounce was corrected so lastBtn is frozen during the 200 ms debounce window, eliminating the ghost-press bug from Project 4.

EEPROM Frame Layout:

| Frame Index | Start Address | End Address | Usage |
| --- | --- | --- | --- |
| 0 | 0x000 | 0x0FF | Byte 0 = sentinel; bytes 1–255 = payload |
| 1 | 0x100 | 0x1FF | Byte 256 = sentinel; bytes 257–511 = payload |
| 2 | 0x200 | 0x2FF | Byte 512 = sentinel; bytes 513–767 = payload |

Sentinel byte convention: 0xFF = frame free, 0x00 = frame in-use. initFrames() writes 0xFF to all three sentinel bytes in setup() before the scheduler starts, guaranteeing a clean state on every boot regardless of what a previous run may have left in EEPROM.

## CODE

// Project 5 - PROJECT-USERTOS
// EEPROM Data Frames via Counting Semaphore
// Author: Shaun Lamb, CMPE 311 Spring 2026
#include <Arduino_FreeRTOS.h>
#include <semphr.h>
#include <EEPROM.h>
#define LED1_PIN    8
#define LED2_PIN    7
#define FAN_PIN     3
#define BTN_PIN     2
#define FRAME_SIZE  256
#define NUM_FRAMES  3
const int frameAddr[NUM_FRAMES] = {0, 256, 512};
volatile unsigned long interval1 = 0;
volatile unsigned long interval2 = 0;
const int   fanSpeeds[] = {0, 26, 38, 64, 38, 26};
const char *fanLabels[] = {
  "Fan off", "Fan on low", "Fan on medium",
  "Fan on high", "Fan on medium", "Fan on low"};
volatile int  fanStep          = 0;
volatile int  selectedLED      = 0;
volatile bool awaitingInterval = false;
char    inputBuf[16];
uint8_t inputLen = 0;
SemaphoreHandle_t xSerialMutex;
SemaphoreHandle_t xFrameSema;
void Task_Serial(void *pvParameters);
void Task_LED1  (void *pvParameters);
void Task_LED2  (void *pvParameters);
void Task_Fan   (void *pvParameters);
void initFrames() {
  for (int i = 0; i < NUM_FRAMES; i++)
    EEPROM.write(frameAddr[i], 0xFF);
}
int acquireFrame() {
  xSemaphoreTake(xFrameSema, portMAX_DELAY);
  for (int i = 0; i < NUM_FRAMES; i++) {
    if (EEPROM.read(frameAddr[i]) == 0xFF) {
      EEPROM.write(frameAddr[i], 0x00);
      return i;
    }
  }
  xSemaphoreGive(xFrameSema);
  return -1;
}
void releaseFrame(int idx) {
  EEPROM.write(frameAddr[idx], 0xFF);
  xSemaphoreGive(xFrameSema);
}
void writeFrame(int idx, const uint8_t *data, uint8_t len) {
  int base = frameAddr[idx] + 1;
  for (uint8_t i = 0; i < len && i < FRAME_SIZE - 1; i++)
    EEPROM.write(base + i, data[i]);
}
void readFrame(int idx, uint8_t *buf, uint8_t len) {
  int base = frameAddr[idx] + 1;
  for (uint8_t i = 0; i < len && i < FRAME_SIZE - 1; i++)
    buf[i] = EEPROM.read(base + i);
}
void runEepromDemo() {
  int frame = acquireFrame();
  if (frame < 0) { Serial.println("EEPROM: no frame available"); return; }
  static const uint8_t payload[8] = {0xDE,0xAD,0xBE,0xEF,0x01,0x02,0x03,0x04};
  static uint8_t readback[8];
  writeFrame(frame, payload, 8);
  readFrame(frame, readback, 8);
  releaseFrame(frame);
  Serial.print("EEPROM frame "); Serial.print(frame); Serial.print(" readback: ");
  for (int i = 0; i < 8; i++) {
    if (readback[i] < 0x10) Serial.print("0");
    Serial.print(readback[i], HEX); Serial.print(" ");
  }
  Serial.println();
}
void setup() {
  pinMode(LED1_PIN, OUTPUT); pinMode(LED2_PIN, OUTPUT);
  pinMode(FAN_PIN,  OUTPUT); pinMode(BTN_PIN,  INPUT);
  analogWrite(FAN_PIN, 0);
  Serial.begin(9600);
  initFrames();
  while (Serial.available() == 0) { ; }
  while (Serial.available() > 0)  { Serial.read(); }
  Serial.println("Initial state: Button un-pressed");
  Serial.println("Initial state: Fan off");
  Serial.println("");
  Serial.println("What LED? (1 or 2)");
  Serial.println("(type 'e' to run EEPROM frame demo)");
  xSerialMutex = xSemaphoreCreateMutex();
  xFrameSema   = xSemaphoreCreateCounting(NUM_FRAMES, NUM_FRAMES);
  xTaskCreate(Task_Serial, "Serial", 150, NULL, 2, NULL);
  xTaskCreate(Task_LED1,   "LED1",    80, NULL, 1, NULL);
  xTaskCreate(Task_LED2,   "LED2",    80, NULL, 1, NULL);
  xTaskCreate(Task_Fan,    "Fan",     80, NULL, 1, NULL);
}
void loop() {}
void Task_Serial(void *pvParameters) {
  (void) pvParameters;
  for (;;) {
    if (Serial.available() > 0) {
      char c = Serial.read();
      if (c == '\n' || c == '\r') {
        if (inputLen > 0) {
          inputBuf[inputLen] = '\0'; inputLen = 0;
          if (xSemaphoreTake(xSerialMutex, portMAX_DELAY) == pdTRUE) {
            if (inputBuf[0] == 'e' || inputBuf[0] == 'E') {
              runEepromDemo();
              Serial.println("What LED? (1 or 2)");
            } else {
              int value = atoi(inputBuf);
              if (!awaitingInterval) {
                selectedLED = value;
                if (selectedLED == 1 || selectedLED == 2) {
                  Serial.println("What interval (in msec)?");
                  awaitingInterval = true;
                } else {
                  Serial.println("Invalid LED. Enter 1 or 2.");
                  Serial.println("What LED? (1 or 2)");
                }
              } else {
                unsigned long ms = (unsigned long)value;
                if (ms > 0) {
                  if (selectedLED == 1) interval1 = ms;
                  else                  interval2 = ms;
                  Serial.println("What LED? (1 or 2)");
                }
                awaitingInterval = false;
              }
            }
            xSemaphoreGive(xSerialMutex);
          }
        }
      } else { if (inputLen < 15) inputBuf[inputLen++] = c; }
    }
    vTaskDelay(1);
  }
}
static void blinkTask(int pin, volatile unsigned long &interval) {
  for (;;) {
    unsigned long iv = interval;
    if (iv > 0) {
      digitalWrite(pin, HIGH); vTaskDelay(pdMS_TO_TICKS(iv / 2));
      digitalWrite(pin, LOW);  vTaskDelay(pdMS_TO_TICKS(iv / 2));
    } else { digitalWrite(pin, LOW); vTaskDelay(pdMS_TO_TICKS(10)); }
  }
}
void Task_LED1(void *pvParameters) { (void)pvParameters; blinkTask(LED1_PIN, interval1); }
void Task_LED2(void *pvParameters) { (void)pvParameters; blinkTask(LED2_PIN, interval2); }
void Task_Fan(void *pvParameters) {
  (void) pvParameters;
  bool lastBtn = LOW, debouncing = false;
  TickType_t debounceStart = 0;
  for (;;) {
    bool btn = digitalRead(BTN_PIN);
    if (!debouncing && btn == HIGH && lastBtn == LOW) {
      fanStep = (fanStep + 1) % 6;
      analogWrite(FAN_PIN, fanSpeeds[fanStep]);
      if (xSemaphoreTake(xSerialMutex, portMAX_DELAY) == pdTRUE) {
        Serial.println(fanLabels[fanStep]);
        xSemaphoreGive(xSerialMutex);
      }
      debouncing = true; debounceStart = xTaskGetTickCount();
    }
    if (debouncing && (xTaskGetTickCount() - debounceStart) >= pdMS_TO_TICKS(200)) {
      debouncing = false; lastBtn = btn;
    } else if (!debouncing) { lastBtn = btn; }
    vTaskDelay(pdMS_TO_TICKS(10));
  }
}

# TESTING AND VALIDATION

All tests below were conducted on the UNO R3 hardware using two LEDs, 330 Ω resistors, and the motor/fan circuit from Project #4.

Regression tests (carried forward from Project #4):

T.1 – Dialogue from serial monitor sample matches Table 4.

T.2 – Blink rate of each LED is independent and doesn’t change until prompted.

T.3 – Blink rate is specified in milliseconds.

T.4 – Blink rate must be correct.

T.5 – The blink rate setting must be able to repeat at least 5 times.

T.6 – Button press sequence matches: OFF > LOW > MEDIUM > HIGH > MEDIUM > LOW > OFF.

T.7 – Fan speed changes are reflected correctly in the serial monitor after each button press.

T.8 – Fan operation does not interfere with LED blinking or serial monitor dialogue.

T.9 – Fan LOW speed produces minimal airflow and sound.

T.10 – Fan MEDIUM speed produces moderate airflow and sound.

T.11 – Fan HIGH speed produces distinct airflow.

T.12 – Button sequence must be able to repeat at least 3 full cycles.

New tests for EEPROM data frames (E.4.5):

T.13 – Typing 'e' in the serial monitor triggers the EEPROM demo and prints frame index and readback bytes (DE AD BE EF 01 02 03 04).

T.14 – Typing 'e' multiple times cycles through frames 0, 1, and 2 as the semaphore tracks availability.

T.15 – After the demo completes, the semaphore count returns to 3 and no frames are permanently held.

T.16 – EEPROM demo does not interfere with LED blinking, fan button, or serial input flow.

Results:

| Test | Description | Result |
| --- | --- | --- |
| T.1 | Dialogue from serial monitor matches Table 4 | PASS |
| T.2 | LED blink rates are independent | PASS |
| T.3 | Blink rate specified in milliseconds | PASS |
| T.4 | Blink rate is correct | PASS |
| T.5 | Blink rate setting repeats at least 5 times | PASS |
| T.6 | Button cycles OFF > LOW > MED > HIGH > MED > LOW > OFF | PASS |
| T.7 | Fan speed reflected in serial monitor after each press | PASS |
| T.8 | Fan does not interfere with LEDs or serial | PASS |
| T.9 | Fan LOW produces minimal airflow and sound | PASS |
| T.10 | Fan MEDIUM produces moderate airflow and sound | PASS |
| T.11 | Fan HIGH produces distinct airflow | PASS |
| T.12 | Button sequence repeats at least 3 full cycles | PASS |
| T.13 | 'e' command triggers EEPROM demo and prints correct readback | PASS |
| T.14 | Multiple 'e' commands cycle frames via semaphore | PASS |
| T.15 | Semaphore count returns to 3 after demo; no frames held | PASS |
| T.16 | EEPROM demo does not interfere with other tasks | PASS |

# VIDEO

https://youtube.com/shorts/Bv-NnhXEvI8?si=9vk6mTFgtqG7FljW

(Project 4 video – replace with Project 5 video link before submission)